|  | BMS COLLEGE OF ENGINEERING, BANGALORE-19 (Autonomous Institute, Affiliated to VTU) Department of Electronics and Communication |
| --- | --- |
| UNIT III Question Bank Topics: Oscillators and Op-Amp |  |

PART-I (REMEMBER/UNDERSTAND)

Key Words: name, list, describe, recognize, explain, summarize, Label, match

| No. | Question | Marks |
| --- | --- | --- |
| 1 | What do you mean by electronic oscillator? Name the different types of oscillators. | 1+4 |
| 2 | Describe basic principle of oscillator with a block diagram? Is an external input signal necessary for the output of an oscillator? | 4+1 |
| 3 | Draw the circuit diagram of Hartley oscillator and explain its operation. | 2+3 |
| 4 | How sustained oscillations are produced in crystal oscillator and explain its operation with a diagram. | 3+2 |
| 5 | Explain RC phase shift oscillator with a neat circuit diagram and state any two applications of it. | 2+2+1 |
| 6 | Draw the equivalent circuit of crystal and explain the Piezoelectric property. Write the expressions for series parallel resonance frequencies. | 2+2+1 |
| 7 | Explain the block diagram of differential amplifier showing the four major stages, and list any four applications of Op-Amp | 3+2 |
| 8 | Draw the circuit of BJT differential amplifier and explain. Define CMRR and how it can be increased? | 3+2 |
| 9 | Explain the block of operational amplifier with a neat diagram. | 3+2 |
| 10 | - What is an Operational Amplifier? - Mention four applications of Op-Amp - Give four advantages of using Op-Amp | 1+2+2 |
| 11 | - State the characteristics of an Ideal Op-Amp - How do the characteristics of a practical Op-Amp differ from those of the ideal Op-Amp? | 3+3 |
| 12 | Draw the circuit diagram of an Inverting amplifier using an Op-Amp. “A virtual ground exists at the input of the proper amplifier” - Explain | 2.5+2.5 |
| 13 | Draw the circuit of voltage follower and explain, Mention any two applications in communication. “The current entering into the Op-Amp is ideally zero”- Explain | 4+1+1 |
| 14 | Describe the operational amplifier as an integrator | 05 |
| 15 | Describe the operational amplifier as an differentiator | 05 |

PART-II (APPLY)

Key Words: derive, solve, develop, and obtain (maps to PO1)

| No. | Question | Marks |
| --- | --- | --- |
| 1 | - The parameters of a crystal fitted in a crystal oscillator are as follows: L = 0.4H, C = 0.08pF, C1 = 1pF and R = 6kΩ. Determine - The series resonant frequency - The parallel resonant frequency | 06+04 |
|  | - Estimate the values of R and C for an output frequency of 2kHz in a RC phase shift oscillator. |  |
| 2 | - What is the frequency of Hartley oscillator if the total inductance of L1 and L2 is 50mH and the capacitance of the resonant circuit is 200pF? | 04+06 |
|  | - In a phase shift oscillator that uses three RC sections, RL = R = 10 kΩ. If the oscillator is to generate frequencies in the range from 1 to 100 kHz, what should be the range of C? |  |
| 3 | - The transistor RC phase shift oscillator has three RC sections with C = 0.01µF and R = 6.3 KΩ. If RL = 4.7kΩ. Determine the frequency of oscillation. | 04+06 |
|  | - The equivalent electrical parameter of a crystal oscillator are L = 3H, C = 0.05pF, CP = 6pF, and R = 1KΩ. Determine the series resonant frequency and the percentage by which the parallel resonant frequency exceeds the series resonance frequency. |  |
| 4 | - Derive the expression of the output of an Op-Amp Integrator with a neat circuit diagram. | 05+05 |
|  | - Draw the output waveform of an Op-Amp - If square wave is given at the differentiator input. - If triangular wave is given at the differentiator input |  |
| 5 | - For the following circuit , find the value of Vo with input resistance=1kΩ , Feedback resistance = 10KΩ and input voltage=2mV | 06+04 |
|  | - An Op-Amp non-inverting amplifier has an input resistance of 10KΩ and Feedback resistance of 50KΩ. If the input voltage is 0.5V, find the output voltage and input current. |  |
| 6 | - Derive the output expression of an operational amplifier as a summing amplifier and averaging circuit. - Design an Op-Amp inverting amplifier of voltage gain -10 and input resistance 10KΩ. The circuit should amplify signals in the range -1.5V to +1.5V. What should be the minimum Op-Amp supply voltages? | 05+05 |
| 7 | - Derive how Operational Ampifier can be used as a subtracting device. | 06+04 |
|  | - For the following Op-amp circuit, Rin = 1000Ω and gain A = -75. Determine the value of feedback resistance.(VCC = 12V and –VEE = -12V). |  |
| 8 | - Consider an ideal Op-Amp circuit as shown in below figure. Given that R1 = 10 KΩ, Rf = 30KΩ, VS = 4V, find Vo | 06+04 |
|  | - For the following Op-amp circuit, Rg = 12KΩ and gain A = 50. Determine the value of feedback resistance. (VCC = 12V and –VEE = -12V). |  |
| 9 | - Derive the expression for output voltage of a differential amplifier | 05+05 |
|  | - The summing amplifier as shown in below figure has Rf = 10KΩ, R1 = 10KΩ, R2 = 2.2KΩ and R3 = 3.3KΩ, If V1 = 6V, V2 = -3V, V3 = -0.75V, Find Vo |  |
| 10 | - Determine the output voltage of a differential amplifier for input voltages of 320µV and 260µV. The differential gain of the amplifier is 4000 and the value of CMRR is 200 | 04+06 |
|  | - In the op-amp circuit shown below, evaluate V0 as a function of V1, V2 and V3 |  |
| 11 | - Derive the output expression of inverting and non-inverting amplifier | 06+04 |
|  | - A differential amplifier has a typical common mode gain of 40dB and CMRR of 80dB. Find the output voltage Vo when the input voltages are 0.20mV and 0.25mV. |  |
| 12 | - Consider the operational amplifier circuit as shown below. Assuming it to ideal the output voltage is given by Vo = A1V1+A2V2. Find the values of constants A1 and A2 for the resistor combination R1 = 10KΩ, R2 = 100KΩ, R3 = 99KΩ. | 06+04 |
|  | - Derive the expression for output voltage of an Op-Amp differentiator with a diagram |  |

PART-III (DESIGN/ANALYZE/ENGINEERING TOOL/ANY OTHER)

ANALYSE: conclude, classify, infer, execute, maximize, minimize,

(Questions from GATE/IES/CAT/any other)

DESIGN: build, model, design

| No. | Question | Marks |
| --- | --- | --- |
| 1 | Design an inverting summer using op-amp with the resistance values of Rf=90KΩ, R1=1KΩ, R2=10KΩ and R3=100kΩ. Calculate its output voltage in terms of its input voltages V1, V2 and V3. | 05 |
| 2 | Design an Op-Amp circuit to provide an output of Vo= - [3V1+1/2V2]. | 05 |
| 3 | Calculate the output voltage of the circuit shown in figure, if V1= - 0.2V and V2=0V. | 05 |
| 4 | Determine the output voltage for the circuit shown in figure with a sinusoidal input of 2.5 mV. | 05 |
| 6 | Calculate the output voltage of the circuit shown in figure, if V1 = 300 mV and V2 = 700 mV. Also mention about how the op-amp is configured. | 05 |
| 8 | Calculate the output voltage Vo for the circuit shown in figure, if V1 = –V2 = 300 mv | 05 |
| 9 | Identify the circuit shown in figure. When a sine wave of 1vpeak at 1000Hz is applied to the circuit with the following specification: RF =1kΩ and C1=0.33µF, find its output waveform and its output equation. | 05 |
| 10 | Choose the value of RF and C for a 5kHz input signal to obtain good differentiation. a) RF = 1.6×103Ω, C1 = 33×10-6F b) RF = 1.6×103Ω, C1 = 0.47×10-6F c) RF = 1.6×103Ω, C1 = 47×10-6F d) RF = 1.6×103 Ω, C1 = 10×10-6F.Also justify your answer with calculation. | 05 |
| 11 | Design an appropriate circuit using Op-amp, whose input and output signals relations are as shown in figure. Also justify your answer with relevant equations. | 05 |
| 12 | Design a single op-amp circuit that performs amplified average function with a gain of 10. Draw the circuit diagram and clearly mark all the resistors values. | 05 |